| Ex.No.2 | Study of Network cables. - Understand different types of Network cables. - Make a cross-wired cable and straight through cable using clamping/crimping tool. |
| --- | --- |
| Date: |  |

a) 	Understand different types of network cable.

Different type of cables used in networking are:

  - Unshielded Twisted Pair (UTP) Cable

  - Shielded Twisted Pair (STP) Cable

  - Coaxial Cable

  - Fibre Optic Cable

| Cable type | Category | Maximum Data Transmission | Advantages/ Disadvantages | Application/Use |  |  |  | Image |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UTP | Category 3 | 10 bps | Advantages - Cheaper in cost - Easy to install as they have a smaller overall diameter. Disadvantages - More prone to (EMI) Electromagneti c interference and noise | 10Base-T Ethernet Fast Ethernet, Gigabit Ethernet Fast Ethernet, Gigabit Ethernet |  |  |  |  |
|  | Category 5 | Up to 100 Mbps |  |  |  |  |  |  |
|  | Category 5e | 1Gbps |  |  |  |  |  |  |
| STP SSTP | Category6,6a | 10Gbps 10Gbps | Advantages ● Shielded. - Faster than UTP. - Less susceptible to noise and interference Disadvantages - Expensive - Greater installatio n effort | Gigabit Ethernet, 10G Ethernet (55m) Widely used in data centres |  |  |  |  |
|  | Category 7 |  |  |  |  |  |  |  |
|  |  |  |  | Gigabit Ethernet, |  |  |  |  |
|  |  |  |  | 10G Ethernet |  |  |  |  |
|  |  |  |  | (100m) |  |  |  |  |

| Coaxial cable | RG-6 RG-59 RG-11 | 10-100Mbps | - High bandwidth - Immune to interferen ce - Low loss bandwidt h - Versatile - Disadvantages - Limited distance - Cost - Size is bulky | Speed of signal is 500m Televisi on network High speed internet connect ions |  |
| --- | --- | --- | --- | --- | --- |
| fibre optics cable | Single mode Multi mode | 100Gbps | Advantages - High speed - High bandwidth - High security - Long distance Disadvantages - Expensive - Requires skilled installers | ● Maximum distance of fibre optics cable is around 100meters |  |

(b) Make Your Own Ethernet Cross-Over Cable/ Straight cable

Tools and parts needed:

- Ethernet cabling. CAT5e is certified for gigabit support, but CAT5 cabling works as well, just over shorter distances.

- A crimping tool. This is an all-in-one networking tool shaped to push down the pins in the plug and strip and cut the shielding off the cables.

- Two RJ45 plugs.

- Optional two plug shields.

- Ethernet Cable (CAT5e/CAT5)

  - CAT5e is recommended because it supports Gigabit Ethernet (1000 Mbps).  ● 	CAT5 also works but supports lower speeds and shorter distances.

- Crimping Tool

  - A networking tool used to:

o Strip the outer insulation of the cable.  o Arrange and trim the wires.  o Crimp (press) the RJ45 connector onto the cable.    RJ45 Connectors (2)

  - Plastic connectors fitted at both ends of the Ethernet cable.

  - Used to connect the cable to computers, switches, routers, etc.

- Plug Shields (Optional)

  - Protective covers placed over the RJ45 connectors.

  - Prevent damage to the locking clip and reduce strain on the cable.