Ex. No. 4(a) – Setup and Configure a LAN Using a Switch

Aim

To set up and configure a Local Area Network (LAN) using a network switch and Ethernet cables, assign IP addresses, verify connectivity, and perform file sharing.

Requirements

4 PCs

1 Network Switch

4 Ethernet/UTP cables

Cisco Packet Tracer

Part A – Physical LAN Setup

Procedure

Connect four computers to the switch using Ethernet cables and ensure the link LEDs are ON.

Configure the IP addresses and subnet mask as follows:

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| PC1 | 10.1.1.1 | 255.0.0.0 |
| PC2 | 10.1.1.2 | 255.0.0.0 |
| PC3 | 10.1.1.3 | 255.0.0.0 |
| PC4 | 10.1.1.4 | 255.0.0.0 |
| Switch | 10.1.1.5 | 255.0.0.0 |

Open Command Prompt and use the ping command to test connectivity between the computers.

Create a folder, enable sharing, select Everyone, and complete the sharing process.

From another PC, access the shared folder using the computer's IP address or name.

Observation

Four PCs were successfully connected to the switch. IP addresses were assigned correctly. Connectivity between the PCs was verified using ping, and the shared folder was successfully accessed.

Result

Thus, a physical LAN was successfully configured and communication and file sharing between the computers were verified.

Part B – LAN Configuration Using Cisco Packet Tracer

Aim

To design and configure a LAN using Cisco Packet Tracer and verify communication between the connected devices.

Requirements

Cisco Packet Tracer

1 Cisco 2960 Switch

4 PCs

Copper Straight-Through cables

Procedure

Open Cisco Packet Tracer, place one 2960 switch and four PCs, and rename them PC0, PC1, PC2 and PC3.

Connect each PC to the switch using Copper Straight-Through cables.

Configure the following IP addresses with subnet mask 255.0.0.0. Leave the Default Gateway blank.

| PC | IP Address | Subnet Mask |
| --- | --- | --- |
| PC0 | 10.1.1.1 | 255.0.0.0 |
| PC1 | 10.1.1.2 | 255.0.0.0 |
| PC2 | 10.1.1.3 | 255.0.0.0 |
| PC3 | 10.1.1.4 | 255.0.0.0 |

Open Command Prompt and use ipconfig to verify the IP configuration.

Use ping to test connectivity between all PCs.

Use Simulation Mode and Simple PDU to observe ICMP Echo Request and Echo Reply packets travelling through the switch.

Add a server, assign IP address 10.1.1.5 with subnet mask 255.0.0.0, and enable FTP service.

Create an FTP username and password.

From PC0, connect using ftp 10.1.1.5 and use dir, get, and put commands to verify file transfer.

Save the Packet Tracer project as LAN_Configuration.pkt.

Observation

Four PCs were connected to the switch successfully. IP addresses were configured correctly. Ping confirmed communication between the PCs, ICMP packets were observed in Simulation Mode, and FTP file upload and download were successfully verified.

Result

Thus, a LAN was successfully designed and configured in Cisco Packet Tracer using a switch, and network connectivity and file transfer were successfully verified.